Cellular and Personal Communications Systems

- Bell Labs introduced the first mobile system in 1946, called Mobile Telephone Service (MTS).

- Operating at half-duplex, only one person could speak at a time.

- Extremely limited number of channels available.

- An operator was necessary for connecting customers.

- In 1965 Bell Systems introduced Improved Mobile Telephone Service (IMTS).

- Full duplex system allowing simultaneous 2-way conversations.

- Automated system allowed users to dial directly.

- Cell locations were high output power stations allowed a single radio location to serve an entire city.

- Required two radio channels, one for transmit, and one for receive.

- Locations were designed to cover large geographic areas up to 50 miles in diameter.

- In 1970, AT&T proposed to build a high-capacity cellular telephone system called AMPS – for Advanced Mobile Phone Service.

- Chicago was selected as the first test city.

- The FCC set aside 666 new radio frequencies for land-to-mobile communications.

- The frequencies were divided into two equal bands of 333 each, called band “A”, and band “B.”

- This was later increased to 832 total cellular channels of which 416 were allocated to each band.

- The FCC decreed that one band in each city would be licensed to the local exchange company, and one would be licensed to a “non-wireline” carrier.

- The first commercial system began operating in Chicago in 1983 via a call to a descendant of Alexander Graham Bell from Soldier Field.

- The FCC divided the US into 734 cellular markets.

- 306 large geographic areas containing a population of 150,000 or more were defined as Metropolitan Statistical Areas, or MSA’s.

- 428 Rural Service Areas (RSA’s) were established.

But 416 channels in one market is not much.  Especially since several channels are used for setting up a call.

- Cellular was born to “re-use” frequencies across cities and the country over and over again.

- This greatly increased the availability to support many concurrent conversations at a time.

- Cellular system designs are based on a hexagon grid of many low-powered radio stations.

- Radio coverage is actually “circular” (more like blobs) in nature, however, the use of the hexagon grid system makes planning easier to visualize.

- Cell splitting – Split original cell into four cells, or more commonly, the original cell is shrunk and six or so new cells are grown around the original.

- Handoff – Moving from one cell to another, the system assigns you to another two channels (more on this later).

- Cellular Telephone System Components

- Mobile Stations or Telephones

- Base Transceiver Station

- Actual radio tower location serving the phones.

- Either the first or last transmission leg of every cellular telephone call.

- Base Station Controller is building next to BTS

- Mobile Telephone Switching Offices (MTSO’s)

- The switch that serves a cellular system.

- Similar in function to a class 5 central office switch.

- Primary purpose is to provide a voice path connection between a mobile phone and a land-based phone, or between two mobile phones.

- Numerous components support the MTSO, such as Home Location Register, Visitor Location Register, and Authentication Controller

Mobile Telephone Switching Office (MTSO) Call Processing

- Mobile Identification Number (MIN)

- 34-bit binary number that translates to the phones 10-digit PSTN number.

- Programmed by the supplier when obtaining service.

- Electronic Serial Number (ESN)

- 32-bit binary number that is programmed at the factory.

- Contains an 18-bit serial number and an 8-bit manufacturer number.

- 4 billion different combinations.

- Embedded in a chip and cannot be changed.

- Mobile-Originated Call

- When a mobile phone is turned on, it monitors the 21 possible control channels within its band.

- Determines the strongest available signal and “locks on.”

- Many systems support registration, in which mobile phone identifies itself to MTSO when first turned on.

- Sends call origination data to MTSO on uplink control channel to the closest cell site.

- Dialed number

- MIN

- ESN

- Base station receives call origination message and forwards it to the MTSO.

- The MTSO verifies the MIN/ESN combination and seizes a trunk on the PSTN interconnect.

- The MTSO allocates a traffic channel at the original cell site to carry the call.

- The cell base station sends supervisory audio tone (SAT) to the mobile phone by adding it to voice signal prior to use.

- The cell base station sends the frequency of the SAT of the traffic channel that will be used to the mobile phone via the control channel.

- The mobile phone tunes into that frequency and looks for the SAT.

- Upon successful receipt of the SAT, the mobile phone retransmits the SAT to the base station.

- The cell base station detects the regenerated SAT and sends an “origination complete” message to the MTSO.

- The MTSO connects the PSTN to the traffic channel that was allocated for this call.

- Autonomous Mobile Registration

- All cellular phones continually transmit their MIN/ESN combination to the nearest cell site every 5-15 seconds.

- This allows the MTSO to know to which cell base station to route incoming calls for the mobile phone.

- Mobile-Terminated Call

- Someone dials your 7-digit mobile telephone number from a land-based phone.

- The PSTN routes the call to the assigned MTSO for this number.

- The MTSO checks its database to determine the area that this phone was in last, and broadcasts a “page” to all base stations in that area.

- The base stations receive the page message and transmit it on the downlink control channel.

- The mobile telephone receives all page messages while monitoring the control channel.  Upon receiving a page message containing its MIN, it transmits the phones MIN and ESN on the uplink control channel.

- The base station forwards the MIN/ESN combination to the MTSO to tell it where the mobile phone is.

- The MTSO verifies the MIN/ESN combination and allocates a traffic channel at the original cell site to carry the call.

- The cell base station sends supervisory audio tone (SAT) to the mobile phone by adding it to voice signal prior to use.

- The cell base station sends the frequency of the SAT of the traffic channel that will be used to the mobile phone via the control channel.

- The mobile phone tunes into that frequency and looks for the SAT.

- Upon successful receipt of the SAT, the mobile phone retransmits the SAT to the base station.

- The cell base station detects the regenerated SAT and sends an “alert” order to the mobile telephone to make it “ring.”

- The mobile station confirms the alert status by retransmitting the signaling tone back via the traffic channel.

- When the customer answers the phone, it ceases the retransmission of the signaling tone back to the cell base station.

- The base station detects that the signaling tone has stopped, and sends a message to the MTSO to inform it that the customer has answered the call.

- The MTSO connects the PSTN to the traffic channel that was allocated for this call.

- Call Handoff

- Cell base stations continually monitor the signal strength being received from the mobile phone, Received Signal Strength Indicator (RSSI).

- When the signal strength drops below the manually set threshold, a call handoff is justified.

- The cell base station sends a handoff request to the MTSO and includes the mobile telephone’s current power level and signal strength.

- The MTSO receives the request and determines what other cell stations are adjacent to the cell station currently handling the call.

- The MTSO sends a handoff measurement request to an adjacent cell station, cell base station B.

- Base station B tunes to the mobile telephone’s transmitting channel and determine the signal strength.

- If the measurement exceeds the criteria for handoff, then cell base station B will transmit the measurement to the MTSO.

- The MTSO receives the handoff measurement and chooses a transmit channel for cell base station B to use for the call.

- The MTSO sends the transmit channel assignment to station B, who readies the channel and acknowledges the MTSO.

- The MTSO receives the confirmation message from station B and sends a handoff order message to the original cell base station containing the frequency and SAT of the new transmit channel in base station B.

- The original cell base station transmits the handoff order to the mobile telephone.

- The mobile telephone hears the handoff order, confirms the order, and re-tunes its frequency synthesizer for the transmit channel in base station B.

- The mobile telephone listens for the supervisory audio tone being transmitted from base station B, and upon receiving it, retransmits it back to base station B.

- Base station B receives it, and sends a handoff ok message to the MTSO.

- The MTSO sends a release source channel message to the original cell base station, which releases the transmit channel it was using for the call.

###### Various Wireless Systems

Advanced Mobile Phone Service (AMPS) – IS-54

- Uses frequency division multiplexing, analog signals

- Bandwidth is divided into 30 kHz channels.

- Each channel is used for either transmitting or receiving.

- This technology is almost dead in North America

- AMPS Cellular Frequencies:

- Mobile Transmit: 	824 MHz – 849 MHz

- Mobile Receive: 	869 MHz – 894 MHz

Digital Advanced Mobile Phone Service (D-AMPS) – IS-136

- Uses TDMA to break up each AMPS channel into 3 separate channels

- Same frequency ranges as AMPS

- Additional features such as caller ID and longer battery life

- Cingular Wireless and AT&T both still offer D-AMPS

# Code Division Multiple Access – IS-95a

- Also known as TIA/EIA-95

- Based on direct sequence spread spectrum technology

- Developed by engineers at Qualcomm

- Verizon Wireless, PrimeCo, and Sprint PCS use this technology

- Uses same frequency ranges as AMPS and D-AMPS

# GSM – Global System for Mobile Communications

- A TDMA technology

- Widely popular in Europe but minimal in North America

- Cingular Wireless and Voicestream use GSM

- Uses 880 – 915 MHz and 925 – 960 MHz

iDEN (Integrated Dispatch Enhanced Network)

- Functionally the same as MIRS (Motorola Integrated Radio System)

- A high-capacity digital trunked radio system providing integrated voice and data services to its users

- Used by Nextel Communications

GPRS (General Packet Radio Service)

- The 2.5G version of GSM

- Theoretically allows each user access to 8 GSM data channels at once, boosting data transfer speeds to more than 100Kbps (30Kbps in the real world since it only uses 2 GSM channels)

UMTS (Universal Mobile Telecommunications System)

- Also called Wideband CDMA

- The possible 3G version of GPRS

- UMTS is not backward compatible with GSM, so first UMTS phones will have to be dual-mode

- Based on TDMA, same as D-AMPS and GSM

1xRTT (CDMA2000 1x Radio Transmission Technology)

- The 2.5G backwards compatible replacement for CDMA

- 1xRTT will replace GPRS

- 1x means that it requires only the same amount of spectrum as 2G networks based on CDMA (IS-95)

1xEV (1x Enhanced Version)

- The 3G replacement for 1xRTT

- Will come in two flavors: 1xEV-DO for data only, and 1xEV-DV for data and voice

# EDGE (Enhanced Data rates for Global Evolution)

- Further upgrade to GSM

- Possible 3G replacement for GPRS

- Uses improved modulation to triple the data rate where reception is clear

| Company | Claimed U.S. Coverage | U.S. Customers | 2G | 2.5G (mid 2002) | 3G (late 2003) |
| --- | --- | --- | --- | --- | --- |
| Verizon Wireless | 90% | 28 million | IS-95a (CDMA) | 1xRTT | 1xEV |
| Cingular Wireless | 70% | 22 million | GSM and D-AMPS | GPRS | EDGE or UMTS |
| AT&T Wireless | 98% | 18 million | D-AMPS | GPRS | EDGE or UMTS |
| Sprint PCS | 83% | 13 million | IS-95a (CDMA) | 1xRTT | 1xEV |
| Nextel Communic | 77% | 8 million | IDEN | 1xRTT | 1xEV |
| Voicestream | 97% | 6 million | GSM | GPRS | EDGE or UMTS |

From Network Magazine December 2001

General feeling is both 2.5G and 3G systems will not live up to their original hype for data rates (in U.S. and elsewhere).